Microwave Engineering- ECA1308

Student Name: U.Guru Prasad Reddy

Reg no: 192312683L, Date(9/10/26)

Experiment 1: TE10 Mode Simulation: A WR-90 rectangular waveguide has a = 22.86 mm, b = 10.16 mm and length L = 100 mm. Operate it at f = 10 GHz and excite the dominant TE10 mode in HFSS. Obtain the E-field and H-field distributions, simulated cutoff frequency fc and propagation constant β. Calculate fc (theory) = c/(2a), β = (2πf/c)√[1−(fc/f)²], and the percentage deviation of simulated fc from theoretical fc.

Aim: To simulate the dominant TE₁₀ mode in a WR-90 rectangular waveguide using ANSYS HFSS at 10 GHz, and to obtain the E-field distribution, H-field distribution, cutoff frequency, and propagation constant. The simulated cutoff frequency is compared with the theoretical value and its percentage deviation is calculated.

Software Required:MATLAB

PROCEDURE:

- Open MATLAB and create a new Live Script or MATLAB Script from the MATLAB Home window.

- Define the waveguide parameters such as width \(a\), height \(b\), length \(L\), operating frequency \(f\), and speed of light \(c\) in the MATLAB workspace.

- Create the rectangular waveguide cross-section using MATLAB's meshgrid function to generate the \(x\)- and \(y\)-coordinates inside the WR-90 waveguide.

- Set the TE₁₀ mode by defining the electric-field distribution across the waveguide width using the sinusoidal TE₁₀ field pattern.

- Generate the E-field distribution using MATLAB plotting commands such as surf, mesh, or contourf and display the field variation inside the waveguide.

- Generate the H-field distribution corresponding to the TE₁₀ mode and plot it using MATLAB graphical functions.

- Calculate and display the theoretical cutoff frequency and propagation constant using MATLAB commands and show the values in the Command Window.

- Enter the simulated cutoff frequency obtained from HFSS into MATLAB and calculate the percentage deviation between the HFSS result and the theoretical cutoff frequency.

- Display the final results including the E-field plot, H-field plot, theoretical cutoff frequency, simulated cutoff frequency, propagation constant, and percentage deviation, and verify that the TE₁₀ mode propagates at 10 GHz.

Program:

clc;

clc;

clear;

close all;

%% TE10 Mode Simulation - WR90 Waveguide

% Waveguide dimensions

a = 22.86e-3;       % Width in meters

b = 10.16e-3;       % Height in meters

L = 100e-3;         % Length in meters

% Operating frequency

f = 10e9;           % 10 GHz

% Speed of light

c = 3e8;

%% 1. Theoretical Cutoff Frequency

fc = c/(2*a);

fprintf('TE10 Cutoff Frequency = %.4f GHz\n', fc/1e9);

%% 2. Propagation Constant

beta = (2*pi*f/c)*sqrt(1-(fc/f)^2);

fprintf('Propagation Constant = %.4f rad/m\n', beta);

%% 3. Percentage Deviation

% Enter HFSS simulated cutoff frequency here

fc_HFSS = 6.557e9;     % Example HFSS value

deviation = abs((fc_HFSS-fc)/fc)*100;

fprintf('Percentage Deviation = %.4f %%\n', deviation);

%% 4. Waveguide Grid

x = linspace(0,a,100);

y = linspace(0,b,60);

[X,Y] = meshgrid(x,y);

%% 5. TE10 E-field Distribution

% Normalized transverse electric field

Ey = sin(pi*X/a);

figure;

surf(X*1000,Y*1000,Ey);

shading interp;

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('Normalized E-field');

title('TE10 Mode - E-field Distribution');

colorbar;

view(2);

%% 6. TE10 H-field Distribution

Hx = sin(pi*X/a);

figure;

surf(X*1000,Y*1000,Hx);

shading interp;

xlabel('x (mm)');

ylabel('y (mm)');

zlabel('Normalized H-field');

title('TE10 Mode - H-field Distribution');

colorbar;

view(2);

%% 7. Display Results

fprintf('\n===== TE10 RESULTS =====\n');

fprintf('Waveguide width a = %.2f mm\n',a*1000);

fprintf('Waveguide height b = %.2f mm\n',b*1000);

fprintf('Operating frequency = %.2f GHz\n',f/1e9);

fprintf('Theoretical fc = %.4f GHz\n',fc/1e9);

fprintf('Beta = %.4f rad/m\n',beta);

fprintf('HFSS fc = %.4f GHz\n',fc_HFSS/1e9);

fprintf('Percentage deviation = %.4f %%\n',deviation);

:

Output:

FIG:1

FIG:2

Result:

The WR-90 rectangular waveguide with a = 22.86 mm, b = 10.16 mm was analyzed at 10 GHz for the dominant TE10 mode. The theoretical cutoff frequency was obtained as 6.557 GHz, and the propagation constant was approximately 198.9 rad/m. The MATLAB field plots show the expected TE10 E-field and H-field distributions across the waveguide. Since the operating frequency 10 GHz is greater than the cutoff frequency 6.557 GHz, the TE10 mode propagates successfully.